REPORT TO

FIVE RIVERS METROPARKS

Section 6: Englewood MetroPark

March 3, 2014

Legend of Abbreviations

aff…	   Above the finished ground

AR…	   Accessible Route

CIL…	   Change in Level

CFS…   Clear Floor Space

CGS…   Clear Ground Space

EPC…   Elevated Play Component

GLPC…Ground Level Play Component

Background

The RAC staff visited Englewood MetroPark to conduct an access audit. The findings are below.

There are three format additions in this report.  We included cells for these items: responsible employee (required by regulation), progress towards completion (best practice), and date of completion (required by regulation).  We have added the projected costs of work as a best practice, and costs are found in the transition grid.

NORTH PARK

  - Parking – [CHECKLIST]

Trailer spots: 12 spots, none designated accessible EP310

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.1.1 is integral to compliance with title II program access test): (Trailer Spots)

    - Leave as is, trailer parking, not intended as a standard vehicle lot (EP310)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

Main Lot: not located on shortest route (checklist); 93” first stall, 8’ aisle, needs restriping EP288, EP289; lacks van stall (checklist); slopes- 4% first stall EP290, EP291, 4.6% aisle EP292, EP293, 3.2% second stall EP295; mud and dirt on surface EP299; signs lack violation (checklist); sign heights- 51” EP296, EP297, 44” EP298, EP299; stalls have van signage, but only one stall is van accessible due to aisle placement (checklist); aisle does not connect to AR (checklist)

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.1.1 through 1.1.8 is integral to compliance with title II program access test): (Main Lot)

    - Relocate stalls to be on the shortest AR to the park features (checklist)

    - Repaint stalls and access aisles to be 8’ and 5’ each (EP288, EP289)

    - Repaint van stall and access aisle to 11’ and 5’ or 8' and 8' and remove one of the van signs or relocate access aisle to the passenger side (checklist)

    - Repair or correct slope of parking space and access aisle to max 2.08% in any direction (EP290, EP291, EP292, EP293, EP295)

    - Maintain stalls to be free of mud and other debris (EP299)

    - Acquire and mount correct violation sign for all stalls per state or local statute (checklist)

    - Raise existing accessible parking signs so that lowest end of bottom sign is min 60” aff (EP296, EP297, EP298, EP299)

    - Reconfigure accessible stalls to avoid requiring pedestrians to cross vehicular way, in the alternative, leave as is with striped crosswalk (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

Shelter Lot: slopes- 3% second stall EP300, EP301, 3.1% aisle EP302, EP303, 2.4% first stall EP304, EP305; mud and dirt on surface EP305; signage lacks violation (checklist); signs 44.5” high at both stalls EP306, EP307, EP308, EP309; stalls have

van signage, right stall not large enough (checklist)

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.1.1 through 1.1.5 is integral to compliance with title II program access test): (Shelter Lot)

    - Repair or correct slope of parking space and access aisle to max 2.08% in any direction (EP300, EP301, EP302, EP303, EP304, EP305)

    - Maintain stalls to be free of mud and other debris (EP305)

    - Acquire and mount correct violation sign for all stalls per state or local statute (checklist)

    - Raise existing accessible parking signs so that lowest end of bottom sign is min 60” aff (EP306, EP307, EP308, EP309)

    - Repaint van stall and access aisle to 11’ and 5’ or 8' and 8' (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

Stillwater Access: lacks accessible parking stalls EP330

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.1.1 is integral to compliance with title II program access test): (Stillwater Access)

    - Create one or more 8’ accessible parking stalls, with one 5’ adjacent access aisle, with proper signage and striping (EP330)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

  - Exterior Accessible Route - [CHECKLIST] lacks detectable warnings where walkway meets vehicular way (checklist); cross slopes- 3.3% near parking EP311, EP312, 3.3% at turn EP313, EP314

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.2.1 and 1.2.2 is integral to compliance with title II program access test):

    - Install compliant detectable warning at transitions from walkways to vehicular ways as a smart practice (checklist)

    - Correct or repair sidewalk cross slope along AR to max 2.08% (EP311, EP312, EP313, EP314)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

  - Playground Designated Entry - not applicable

  - Playground Surface/Accessible Route within - not applicable

  - Transfer System - [not applicable

  - Ramps - not applicable

  - Elevated Play Components - not applicable

  - Ground Level Play Components - not applicable

  - Sand box/Play tables - not applicable

  - Park Site - [CHECKLIST]

Garbage Cans (8): all in parking EP317, EP318, EP322

Boat Launch: dirt on level pad, lacks AR to pier EP319, EP320, EP321

Adirondack Chairs (2): lack AR EP317

Grass/Dirt Trails (2): grass and dirt surface EP324, EP325, lack AR (checklist)

Picnic Tables (2): lacks accessible table EP319

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.10.1 through 1.10.6 is integral to compliance with title II program access test):

    - Relocate 20% of garbage cans to be along AR (EP317, EP318, EP322)

    - Maintain boat launch to be free of dirt and debris (EP319, EP320, EP321)

    - Create AR with crushed and compacted stone or similar outdoor material from parking or sidewalk to the boat launch pier (EP319, EP320, EP321)

    - Relocate 20% of Adirondack chairs to be along AR (EP317)

    - Leave as is, existing dirt trails and designate other trails in the system as accessible (EP324, EP325)

    - Replace one picnic table with one with knee and toe clearance, 19” deep at 27” high and 24” deep at 9” high, with a 36” AR around table as a smart practice (EP319)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

  - Shelters/Picnic Areas - [CHECKLIST] 0 of 4 tables accessible EP335; .75” gap EP333, EP334; lacks AR to grill and swinging chairs EP331, EP332

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.11.1 through 1.11.3 is integral to compliance with title II program access test):

    - Replace 20% of the picnic tables with one with knee and toe clearance, 19” deep at 27” high and 24” deep at 9” high, with a 36” AR around the table as a smart practice (EP335)

    - Correct or fill gap on shelter interior (EP333, EP334)

    - Extend AR with crushed and compacted stone or similar outdoor material from shelter to grill and swinging chairs (EP331, EP332)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

  - Park Buildings -

Restrooms: [CHECKLIST] portable toilet in parking lot EP316; 54” x 56” EP315; lacks compliant signage (checklist); not fully compliant (checklist)

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.12.1 is integral to compliance with title II program access test): (Restrooms)

    - Replace portable toilet with compliant model and place along AR (EP316, EP315, checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

  - Other - not applicable

EAST PARK

  - Parking – [CHECKLIST1]

Dam Entry: lacks accessible spaces EP168

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.1.1 is integral to compliance with title II program access test): (Dam Entry)

    - Create one or more 8’ accessible parking stalls, with one 5’ adjacent access aisle, with proper signage and striping (EP168)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

Near Disc Golf Restrooms: stalls not marked, ensure scoping met (checklist); slopes- 4.2% stall EP169, EP170, 4.4% aisle EP171, EP172; no sign EP170

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.1.1 through 1.1.3 is integral to compliance with title II program access test): (Near Disc Golf Restrooms)

    - Create one or more 8’ accessible parking stalls, with one 5’ adjacent access aisle, with proper signage and striping, based on lot capacity (checklist)

    - Repair or correct slope of parking spaces and access aisles to max 2.08% in any direction (EP169, EP170, EP171, EP172)

    - Acquire and mount at appropriate heights and locations accessible parking signs for all the stalls (EP170)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

Disc Golf: lacks accessible spaces EP173, EP174

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.1.1 is integral to compliance with title II program access test): (Disc Golf)

    - Create two or more 8’ accessible parking stalls, with one 5’ adjacent access aisle, with proper signage and striping (EP173, EP174)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

Picnic Areas Along River: stalls not marked, ensure scoping met (checklist); slopes- 4.3% first stall EP176, EP177, 5.5% aisle EP178, EP179, 5% second stall EP180, EP181; lack violation signs (checklist); no AR nearby (checklist)

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.1.1 through 1.1.4 is integral to compliance with title II program access test): (Picnic Areas Along River)

    - Ensure incentive scoping is met based on lot capacity (checklist)

    - Repair or correct slope of parking space and access aisle to max 2.08% in any direction (EP176, EP177, EP178, EP179, EP180, EP181)

    - Acquire and mount violation sign for stalls per state or local statute (checklist)

    - Create AR with crushed and compacted stone or similar outdoor material from parking to site elements (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

Other Unmarked Lots: multiple unmarked lots along river EP175

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.1.1 is integral to compliance with title II program access test): (Other Unmarked Lots)

    - Create one or more 8’ accessible parking stalls, with one 5’ adjacent access aisle, with proper signage and striping based on lot capacity for each lot (EP175)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

[CHECKLIST2]

Near Patty Shelter at Water: gravel lot at water level lacks accessible spaces (checklist); slopes- 3.2% first stall EP182, EP183, 3.1% aisle EP184, EP185, 3.1% second stall EP186, EP187; lacks violation signs (checklist); no AR nearby (checklist)

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.1.1 through 1.1.5 is integral to compliance with title II program access test): (Near Patty Shelter at Water)

    - Create one or more 8’ accessible parking stalls, with one 5’ adjacent access aisle, with proper signage and striping at water level (checklist)

    - Resurface newly created accessible stall at water level with a firm, stable, and slip resistant material (checklist)

    - Repair or correct slope of parking spaces and access aisles to max 2.08% in any direction (EP182, EP183, EP184, EP185, EP186, EP187)

    - Acquire and mount violation sign for stalls per state or local statute (checklist)

    - Create AR with crushed and compacted stone or similar outdoor material from parking o site elements (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

Patty Shelter: one stall labeled van accessible is 10’ and 5’ EP188, EP189; slopes- 5.1% first aisle EP190, EP191, 5.2% first stall EP192, EP193, 4.8% second stall EP194, EP195, 5.1% second aisle EP196, EP197; lacks violation signs (checklist)

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.1.1 through 1.1.3 is integral to compliance with title II program access test): (Patty Shelter)

    - Repaint van stall and access aisle to 11’ and 5’ or 8' and 8' (EP188, EP189)

    - Repair or correct slope of parking spaces and access aisles to max 2.08% in any direction (EP190, EP191, EP192, EP193, EP194, EP195, EP196, EP197)

    - Acquire and mount correct violation sign for all stalls per state or local statute (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

Horse Lot: gravel lot EP198

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.1.1 and 1.1.2 is integral to compliance with title II program access test): (Horse Lot)

    - Create one or more 8’ accessible parking stalls, with one 5’ adjacent access aisle, with proper signage and striping based on lot capacity (EP198)

    - Resurface newly created accessible stalls at water level with a firm, stable, and slip resistant material (EP198)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

Maintenance: gravel lot EP199

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.1.1 and 1.1.2 is integral to compliance with title II program access test): (Maintenance)

    - Create one or more 8’ accessible parking stalls, with one 5’ adjacent access aisle, with proper signage and striping based on lot capacity (EP199)

    - Resurface newly created accessible stalls at water level with a firm, stable, and slip resistant material (EP199)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

  - Exterior Accessible Route - [CHECKLIST] CILs- 1” to disc golf restrooms EP200, EP201, 1” and 1.25” at Patty restroom building EP208, EP209, EP210, EP211; gaps- 2.5” at access aisle to route near Patty restroom EP204, EP205, deterioration near drinking fountain EP206, EP207; 10.4% running slope to disc golf restrooms EP202, EP203

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.2.1 through 1.2.3 is integral to compliance with title II program access test):

    - Repair, bevel, or ramp CIL along AR (EP200, EP201, EP208, EP209, EP210, EP211)

    - Correct or fill gaps along AR (EP204, EP205, EP206, EP207)

    - Correct or repair sidewalk running slope along AR to max 5% (EP202, EP203)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

  - Playground Designated Entry - not applicable

  - Playground Surface/Accessible Route within - not applicable

  - Transfer System - not applicable

  - Ramps - not applicable

  - Elevated Play Components - not applicable

  - Ground Level Play Components - not applicable

  - Sand box/Play tables - not applicable

  - Park Site - [CHECKLIST]

Bench Swings (4): lack AR EP235, EP246, EP250

Info Kiosks (3): lack AR EP236, EP241, EP253

Garbage Cans (11): 2 of 11 compliant, remaining lack AR, some in grass and gravel parking lots (checklist)

Grills (8): lack AR EP238, EP248, EP255

Drinking Fountains (3): single bowl only EP239, EP249, EP254

Disc Golf (18 holes): not designed for wheelchair use, lacks AR EP243

Lake Info Signs (6): mounted low, small text EP240, EP252

Adirondack Benches/Chairs (3): lack AR EP242

Water Pump: not designed for wheelchair use, lacks AR EP245

Nature Play: all elements lack AR, not designed for wheelchair use EP257, EP258, EP259, EP260, EP261, EP262

Small Building: lacks AR, short entry, large gap with water feature EP263

Historical Info Sign: lacks AR EP237

Benches (5): 1 bench compliant near Patty shelter EP256; remaining lack CFS EP244, EP251, EP252, bench at disc golf lacks back support and armrest EP244

Picnic Tables (10): lack accessible tables, in grass EP247, EP255

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.10.1 through 1.10.16 is integral to compliance with title II program access test):

    - Create AR with crushed and compacted stone or similar outdoor material from parking or sidewalk to the bench swings (EP235, EP246, EP250)

    - Create AR with crushed and compacted stone or similar outdoor material from parking or sidewalk to the info kiosks (EP236, EP241, EP253)

    - Relocate 20% of garbage cans to be along AR (checklist)

    - Relocate 20% of grills to be along AR (EP238, EP248, EP255)

    - Replace drinking fountains with hi-lo bowl fountains (EP239, EP249, EP254)

    - Create AR with crushed and compacted stone or similar outdoor material from parking to the first tee of the disc golf course (EP243)

    - Replace existing lake info signs with signage having compliant character heights and placed within proper height and viewing distance of the AR (EP240, EP252)

    - Relocate 20% of Adirondack benches/chairs to be along AR  (EP242)

    - Replace water pump with one of an accessible design and place along the AR (EP245)

    - Create AR with crushed and compacted stone or similar outdoor material from parking or sidewalk to the nature play area (EP257, EP258, EP259, EP260, EP261, EP262)

    - Leave as is the nature play area, and as elements are added do include accessible features that meet the technical requirements for reach range, mounting height, grasping, and are on an accessible route (see final report)

    - Create AR with crushed and compacted stone or similar outdoor material from parking or sidewalk to the small building; leave the interior as is unless dedicated for public use (EP263)

    - Relocate historical info sign to be along AR (EP237)

    - Construct firm, stable, and slip resistant pads at least 36” by 48” adjacent to 20% of benches, and locate along an AR as a smart practice (EP244, EP251, EP252)

    - Acquire and install at least one armrest and backrest to 20% of existing benches as a smart practice (EP244)

    - Replace two picnic tables with ones with knee and toe clearance, 19” deep at 27” high and 24” deep at 9” high, with a 36” AR (EP247, EP255)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

  - Shelters/Picnic Areas - [CHECKLIST]

Patty- 1 of 6 tables accessible design, but in grass EP282; lacks AR to volleyball EP283, horseshoes EP285, strong box EP286, Adirondack chairs/benches EP284; angled supports protrude (checklist)

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.11.1 through 1.11.3 is integral to compliance with title II program access test): (Patty)

    - Relocate accessible table to a location allowing 36” AR around the table as a smart practice (EP282)

    - Extend AR with crushed and compacted stone or similar outdoor material from shelter to volleyball, horseshoes, strong box, and Adirondack chairs/benches (EP283, EP285, EP286, EP284)

    - Raise angled supports in shelter or place cane detectable warning or bollard at foot of supports (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

Horse area- lacks AR EP287

Recommendations (in the alternative, leave as is and designate other system shelters as accessible): (Horse Area)

    - Create AR with crushed and compacted stone or similar outdoor material from parking or sidewalk to shelter (EP287)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | METROPARKS OPTION |

  - Park Buildings - Doors: [CHECKLIST]

Maneuvering Clearance: Patty shelter men’s restroom- CIL at sewer cap EP58; camping pit toilet- small pad EP218, 5.6% slope EP217, EP218, 2” CIL EP215, EP216; horse parking pit toilet- surface EP214

Dimension (32"X80"): women’s Patty storage- 27.5” EP35, EP36; men’s restroom closet- 25” EP59, EP60

Gaps: women’s Patty shelter- deterioration, gaps in manhole cover EP37, EP38, EP39, EP40

Hardware: men’s restroom closet- lock only EP61; horse parking pit toilet- hook lock at 56” EP212, EP213

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.12.1 through 1.12.7 is integral to compliance with title II program access test): (Doors)

    - For doors along public circulation route, repair, bevel, or ramp CILs in doorway maneuvering clearance to max .25” (EP58, EP215, EP216)

    - For doors on public circulation route, enlarge or install cement pad at exit for 18” clearance on latch side, and extend a pathway away from the building for emergency egress (EP218, EP214)

    - For all doors along the public circulation route, correct or repair slope at doorway landing to max 2.08% in any direction for level CFS (EP217, EP218)

    - For all doors along the public circulation route, replace doors with ones having 32" clear width (EP35, EP36, EP59, EP60)

    - For all doors along the public circulation route, replace drain cover at doorways to with one having openings max .5” and resurface deterioration (EP37, EP38, EP39, EP40)

    - For all doors along the public circulation route, install lever hardware where indicated (EP61)

    - For all doors along the public circulation route, remount hardware to be 34” to 48” aff (EP212, EP213)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

Restrooms: [CHECKLIST-MULTIPLE USERS]

Both: signage in wrong location EP134, EP135, EP159; mirror 41” aff EP161, EP162, EP136; pipes lack insulation EP137; rear portion of continuous grab bar lacks 24” on open side EP164, EP140, EP141; grab bar 37” aff EP163, EP164, EP138, EP139; stall door- M not self-closing (checklist), W wall pull side, not self-closing EP142; stall depth for inward opening door- men’s 84” EP166, EP167, women’s 82” (checklist)

Men’s: lacks access symbol (checklist); toilet paper too far EP165

Women’s: lacks toe clearance due to block walls (checklist)

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.12.1 through 1.12.10 is integral to compliance with title II program access test): (Restrooms - Multiple Users)

    - Remount signage for both on wall, latch side of door, 48” to baseline of lowest character and 60” to baseline of highest character (EP134, EP135, EP159)

    - Lower mirrors in both so reflective surface is max 40” aff (EP161, EP162, EP136)

    - Insulate exposed pipes under one sink in both (EP137)

    - Extend rear portion of grab bar 24” to the open side of the toilet and remount bar to 33” to 36” aff in both; in the alternative, install separate side and rear grab bars (EP164, EP140, EP141, EP163, EP164, EP138, EP139)

    - Adjust stall doors in both to be self-closing (checklist)

    - Remove and rehang women’s stall door to open from opposite side for 18” maneuvering clearance on the pull side (EP142)

    - Reverse swing of stall doors in both due to inadequate space in accessible stalls (EP166, EP167, checklist)

    - Acquire and mount signage including access symbol for men’s (checklist)

    - Remount toilet paper dispenser max 7” to 9” from front of toilet, 15” to 48” aff and min 12" above or 1.5" below grab bar in men’s (EP165)

    - Provide toe clearance at the front and side partitions, or enlarge stall to provide the same in women’s (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

[CHECKLIST-SINGLE USER]

Portable Near Patty Shelter: door fails (checklist); lacks compliant signage (checklist); not fully compliant, too small, 24” grab bars, toilet paper high (checklist)

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.12.1 is integral to compliance with title II program access test): (Restrooms - Single User - Portable Near Patty Shelter)

    - Replace portable toilet with compliant model and place along AR (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

Camping Pit Toilet: lacks compliant signage (checklist); centerline 24” EP219, EP220; side grab bar extends 45.5” EP221, EP222; grab bars 37.25” and 38.75” high EP223, EP224, EP225

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.12.1 through 1.12.4 is integral to compliance with title II program access test): (Restrooms - Single User - Camping Pit Toilet)

    - Acquire and mount signage, including Braille and access symbol mounted on wall, latch side of door, 48” to baseline of lowest character and 60” to baseline of highest character (checklist)

    - Remount toilets to 16” to 18” from the side wall to centerline (EP219, EP220)

    - Remount side grab bar to max 12” from the rear wall at the close end and min 54" on the far end, and 33” to 36” aff (EP221, EP222)

    - Remount grab bars in accessible toilet to 33” to 36” aff (EP223, EP224, EP225)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

Horse Parking Pit Toilet: lacks complaint signage (checklist); centerline 19” EP226, EP227; seat height 16” EP228, EP229; side grab bar 30” long EP230, EP231; toilet paper too close EP232; extra toilet paper too high EP233, EP234

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.12.1 through 1.12.6 is integral to compliance with title II program access test): (Restrooms - Single User - Horse Parking Pit Toilet)

    - Acquire and mount signage, including Braille and access symbol mounted on wall, latch side of door, 48” to baseline of lowest character and 60” to baseline of highest character (checklist)

    - Remount toilets to 16” to 18” from the side wall to centerline (EP226, EP227)

    - Replace toilet or seat, or re-set to 17” to 19” aff (EP228, EP229)

    - Replace side grab bar with a 42” long grab bar, mounted max 12” from the rear wall and 33” to 36” aff (EP230, EP231)

    - Remount toilet paper dispenser max 7” to 9” from front of toilet, 15” to 48” aff and min 12" above or 1.5" below grab bar (EP232)

    - Lower extra toilet paper to max 48” aff to be in reach range (EP233, EP234)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

  - Other - Trails [CHECKLIST] gaps in boardwalk exceed .5” EP265, EP266, EP267, EP268

Note: There is not yet a final and enforceable standard regarding trails.  There is guidance, in the form of a final guideline for federally owned and operated sites.

We do recommend that, as a smart practice, MetroParks adheres to the final guideline for trails to the greatest possible extent.

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.13.1 is integral to compliance with title II program access test): (Trails)

    - Correct or repair gaps in boardwalk to be max .5” (EP265, EP266, EP267, EP268)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

Camping [CHECKLIST] lacks AR to all amenities EP271, EP276, EP277; EP270, EP272, EP273, EP274, EP278, EP279, EP280, EP281; Patty’s Hollow and Oak View lack accessible tables EP273, EP275; all three areas lack accessible fire ring EP273, EP275, EP279, EP280

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.13.1 through 1.13.3 is integral to compliance with title II program access test): (Camping)

    - Create AR with crushed and compacted stone or similar outdoor material from parking or sidewalk to the camp amenities at one of the sites (EP271, EP276, EP277, EP270, EP272, EP273, EP274, EP278, EP279, EP280, EP281)

    - Replace 20% of picnic tables with one with knee and toe clearance, 19” deep at 27” high and 24” deep at 9” high, with a 36” AR around table as a smart practice at one of the sites (EP273, EP275)

    - Replace fire pits with compliant units at one of the sites (EP273, EP275, EP279, EP280)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

SOUTH PARK

  - Parking – [CHECKLIST] lacks accessible spaces, needs 2 EP336

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.1.1 is integral to compliance with title II program access test):

    - Create two or more 8’ accessible parking stalls, with one 5’ adjacent access aisle, with proper signage (EP336)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

  - Exterior Accessible Route - [CHECKLIST] lacks AR to south park features (checklist)

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.2.1 is integral to compliance with title II program access test):

    - Create AR with crushed and compacted stone or similar outdoor material from parking or sidewalk to park features (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

  - Playground Designated Entry - not applicable

  - Playground Surface/Accessible Route within - not applicable

  - Transfer System - not applicable

  - Ramps - not applicable

  - Elevated Play Components - not applicable

  - Ground Level Play Components - not applicable

  - Sand box/Play tables - not applicable

  - Park Site - [CHECKLIST]

Garbage Can: lacks AR, in parking EP358

Benches (2): lack CFS EP358, EP359

Picnic Table: lacks accessible table EP358

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.10.1 through 1.10.3 is integral to compliance with title II program access test):

    - Relocate 20% of garbage cans to be along AR (EP358)

    - Construct firm, stable, and slip resistant pads at least 36” by 48” adjacent to 20% of benches, and locate along an AR as a smart practice (EP358, EP359)

    - Replace or add one picnic table with one with knee and toe clearance, 19” deep at 27” high and 24” deep at 9” high, with a 36” AR around table as a smart practice  (EP358)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

  - Shelters/Picnic Areas - not applicable

  - Park Buildings -

Doors: [CHECKLIST]

Maneuvering Clearance: parking pit toilet- surface EP337, EP338, 1” CIL pull side EP339, EP340; camping pit toilet- surface EP341, CIL pull side EP342

Dimension (32"X80"): camping pit toilet- 77.25” EP343

Hardware: camping pit toilet- hook lock EP344

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.12.1 through 1.12.4 is integral to compliance with title II program access test): (Doors)

    - For all doors along the public circulation route, enlarge cement pad at exit to allow 18” clearance on the latch side, and extend a pathway away from the toilet for egress (EP337, EP338, EP341)

    - For all doors along the public circulation route, repair, bevel, or ramp CIL in doorway maneuvering clearance to max .25” (EP339, EP340, EP342 )

    - For all doors along the public circulation route, replace doors with ones having 80" overhead and 32" clear width (EP343)

    - For all doors along the public circulation route, replace hardware with lever hardware where indicated (EP344)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

Restrooms: [CHECKLIST]

Parking Pit Toilet: lacks compliant signage (checklist); centerline 23” EP345, EP346; side grab bar does not extend 54” EP347; side grab bar 36.25” aff EP348, EP349; lacks turning space (checklist)

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.12.1 through 1.12.4 is integral to compliance with title II program access test): (Restrooms - Parking Pit Toilet)

    - Acquire and mount signage, including access symbol, mounted on wall, latch side of door, 48” to baseline of lowest character and 60” to baseline of highest character (checklist)

    - Remount toilets to 16” to 18” from the side wall to centerline (EP345, EP346)

    - Remount side grab bar to max 12” from the rear wall at the close end and min 54" on the far end, and 33” to 36” aff (EP347, EP348, EP349)

    - Ensure 36” clearance next to toilet to provide turning space, in the alternative, renovate pit toilet to allow T-shape turning space next to toilet (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

Camping Pit Toilet: lacks compliant signage (checklist); centerline 22” EP351, EP352; side grab bar extends 47.5” from rear wall EP353, EP354; toilet paper too close EP355; extra toilet paper high EP356, EP357; lacks turning space (checklist)

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.12.1 through 1.12.6 is integral to compliance with title II program access test): (Restrooms - Camping Pit Toilet)

    - Acquire and mount signage, including access symbol, mounted on wall, latch side of door, 48” to baseline of lowest character and 60” to baseline of highest character (checklist)

    - Remount toilets to 16” to 18” from the side wall to centerline (EP351, EP352)

    - Remount side grab bar to max 12” from the rear wall at the close end and min 54" on the far end, and 33” to 36” aff (EP353, EP354)

    - Remount toilet paper dispenser max 7” to 9” from front of toilet, 15” to 48” aff and min 12" above or 1.5" below grab bar (EP355)

    - Lower extra toilet paper to max 48” aff to be in reach range (EP356, EP357)

    - Ensure 36” clearance next to toilet to provide turning space, in the alternative, renovate pit toilet to allow T-shape turning space next to toilet (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

  - Other - Trails [CHECKLIST] mud and dirt trail, no access from parking EP360, EP361, EP362

Note: There is not yet a final and enforceable standard regarding trails.  There is guidance, in the form of a final guideline for federally owned and operated sites.

We do recommend that, as a smart practice, MetroParks adheres to the final guideline for trails to the greatest possible extent.

Recommendations (in the alternative, leave as is and designate other system trails as accessible): (Trails)

    - Leave as is, dirt trail and designate other trails in the system as accessible (EP360, EP361, EP362)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | METROPARKS OPTION |

Camping [CHECKLIST] dirt path connecting elements EP363, EP364; lacks accessible picnic table EP366; garbage can lacks AR EP366; info sign text small EP368; lacks accessible fire ring EP365; firewood box lacks AR EP367

Recommendations (in the alternative, leave as is and designate other system campsites as accessible): (Camping)

    - Create AR with crushed and compacted stone or similar outdoor material to connect elements (EP363, EP364)

    - Replace one picnic tables with ones with knee and toe clearance, 19” deep at 27” high and 24” deep at 9” high, with a 36” AR around table (EP366)

    - Relocate garbage can to be along AR (EP366)

    - Replace existing signage with signage having compliant character heights and placed within proper viewing distance of the AR (EP368)

    - Replace fire ring with a compliant model (EP365)

    - Create AR with crushed and compacted stone or similar outdoor material to firewood box (EP367)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | METROPARKS OPTION |

WEST PARK

  - Parking - [CHECKLIST]

Morgan Shelter- stalls not marked, ensure scoping is met (checklist); portable toilet in first stall EP369; slopes- 3.9% fourth stall EP370, EP371, 3.9% shared aisle 3 and 4 EP372, EP373, 2.6% third stall EP374, EP375, 3% second stall EP376, EP377, 3.9% shared aisle 1 and 2 EP378, EP379, 3.6% first stall EP380, EP381; sign heights- 42.75” first stall EP382, 41.75” second stall EP383, 42” third stall EP384, 39.75” fourth stall EP385, EP386

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.1.1 through 1.1.4 is integral to compliance with title II program access test): (Morgan Shelter)

    - Create one or more 8’ accessible parking stalls, with one 5’ adjacent access aisle, with proper signage and striping based on lot capacity (checklist)

    - Relocate portable toilet to another location out of the accessible stall (EP369)

    - Repair or correct slope of parking space and access aisle to max 2.08% in any direction (EP370, EP371, EP372, EP373, EP374, EP375, EP376, EP377, EP378, EP379, EP380, EP381)

    - Raise existing accessible parking signs so that lowest end of bottom sign is min 60” aff (EP382, EP383, EP384, EP385, EP386)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

Canoes- lacks accessible stalls (checklist)

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.1.1 is integral to compliance with title II program access test): (Canoes)

    - Create one or more 8’ accessible parking stalls, with one 5’ adjacent access aisle, with proper signage and striping based on lot capacity (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

  - Exterior Accessible Route - [CHECKLIST] lacks detectable warning where walkway meets vehicular way (checklist); loose gravel near restroom EP393; cross slopes- 3.3% near Morgan parking EP389, EP390, 5.8% near Morgan restroom EP394, EP395; running slopes- 7.4% near Morgan parking EP387, EP388, 6.7% to Morgan restrooms EP391, EP392

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.2.1 through 1.2.4 is integral to compliance with title II program access test):

    - Install compliant detectable warning at transitions from walkways to vehicular ways as a smart practice (checklist)

    - Maintain AR to be free of loose gravel and other debris (EP393)

    - Correct or repair sidewalk cross slope along AR to max 2.08% (EP389, EP390, EP394, EP395)

    - Correct or repair sidewalk running slope along AR to max 5% (EP387, EP388, EP391, EP392)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

  - Playground Designated Entry - not applicable

  - Playground Surface/Accessible Route within - not applicable

  - Transfer System - not applicable

  - Ramps - not applicable

  - Elevated Play Components - not applicable

  - Ground Level Play Components - [not applicable

  - Sand box/Play tables - not applicable

  - Park Site - [CHECKLIST]

Info Kiosk: lacks AR EP402

Bike Rack: lacks AR EP403

Storage Box: lacks AR (checklist)

Grill: lacks AR EP404

Sledding Hill: not designed for wheelchair use, lacks AR EP407, EP408

Canoe Launch: not designed for wheelchair use, lacks AR EP410, EP411, EP412

Fishing: not designed for wheelchair use, lacks AR EP414, EP415, EP416

Picnic Tables (14): only one accessible design table EP409; remaining not accessible design, all in grass EP405, EP406, EP413

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.10.1 through 1.10.3, and 1.10.6 is integral to compliance with title II program access test):

    - Create AR with crushed and compacted stone or similar outdoor material from parking or sidewalk to the info kiosk, bike rack and storage box (EP402, EP403, checklist)

    - Create AR with crushed and compacted stone or similar outdoor material from parking or sidewalk to the grill (EP404)

    - Create AR with crushed and compacted stone or similar outdoor material from parking or sidewalk to the top of the sledding hill (EP407, EP408)

    - Create AR with crushed and compacted stone or similar outdoor material from parking or sidewalk to the canoe launch (EP410, EP411, EP412); in the alternative, leave as is and designate boat launches at other sites in the system as accessible

    - Create AR with crushed and compacted stone or similar outdoor material from parking or sidewalk to fishing (EP414, EP415, EP416); in the alternative, leave as is and designate fishing at other sites in the system as accessible

    - Replace 20% of picnic tables with one with knee and toe clearance, 19” deep at 27” high and 24” deep at 9” high, with a 36” AR around table as a smart practice (EP409, EP405, EP406, EP413)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO, METROPARKS OPTION |

  - Shelters/Picnic Areas - [CHECKLIST]

Lawwill- 5 benches, lack CFS in shelter EP418; 0 of 5 tables accessible EP424; gravel route EP417; CIL at edges EP420; lacks AR to garbage, checkerboard EP421, bags boards EP422, grills EP419, drinking fountain and strong box (checklist); drinking fountain not accessible design (checklist); angled supports protrude at 70” high at 4” from vertical support EP423, EP424

Recommendations (in the alternative, leave as is and designate other system shelters as accessible): (Lawwill)

    - Provide at least 36” by 48” CFS adjacent to 20% of benches (EP418)

    - Replace one picnic table with one with knee and toe clearance, 19” deep at 27” high and 24” deep at 9” high, with a 36” AR around table as a smart practice (EP424)

    - Create AR with crushed and compacted stone or similar outdoor material from parking or sidewalk to the shelter (checklist)

    - Repair, bevel or ramp CIL at shelter entries (checklist)

    - Extend AR with crushed and compacted stone or similar outdoor material from shelter to the garbage, checkerboard, bags boards, grills, drinking fountain, and strong box (EP421, EP422, EP419, checklist)

    - Replace drinking fountain with a hi-lo bowl fountain (checklist)

    - Raise angled supports in shelter or place cane detectable warning at foot (EP423, EP424)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | METROPARKS OPTION |

Morgan- 0 of 4 tables accessible EP429; lacks AR, grass EP426; lacks AR to horseshoes EP427, volleyball, strong box EP428, drinking fountain near parking EP425; angled supports protrude EP429

Recommendations (in the alternative, leave as is and designate other system shelters as accessible): (Morgan)

    - Replace one picnic table with one with knee, toe clearance, 19” deep at 27” high 24” deep at 9” high, with 36” AR around table as a smart practice (EP429)

    - Create AR with crushed and compacted stone or similar outdoor material from parking or sidewalk to the shelter (EP426)

    - Extend AR with crushed and compacted stone or similar outdoor material from  shelter to horseshoes, volleyball, strong box, and drinking fountain (EP427, EP428, EP425)

    - Raise angled supports in shelter or place cane detectable warning at foot (EP429)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | METROPARKS OPTION |

  - Park Buildings -

Doors: [CHECKLIST]

Maneuvering Clearance: men’s west storage- no entry EP121

Dimension (32"X80"): men’s west storage- 22” EP26

Gaps: men’s west restroom- deterioration EP27, EP28

Hardware: men’s west restroom- small EP25; men’s west storage- lacks hardware (checklist); west women’s restroom- lacks pull side hardware EP55, EP56

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.12.1 through 1.12.4 is integral to compliance with title II program access test): (Doors)

    - Leave as is, maneuvering clearance in closet or storage, correction is technically infeasible (EP121)

    - For doors along the public circulation route, replace doors with doors having 32" clear width (EP26)

    - For doors along the public circulation route, fill and maintain gaps at doorways to max .5” (EP27, EP28)

    - For doors along the public circulation route, replace hardware with lever hardware where indicated (EP25, EP55, EP56, checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

Restrooms: [CHECKLIST-MULTIPLE USERS]

Both: signage in wrong location EP57, EP120, EP133, EP157; mirror 57.5” EP122, EP123, EP143, EP144; pipes lack insulation EP124, EP145; towels high- M 58” EP126, EP127, W 56.5” EP146, EP147; centerline- M 19” EP128, W 18.5” EP148, EP149; side grab bar- M extends 53” EP129, EP130, W extends 48” EP152, EP153; stall door- M not self-closing, slide lock only EP125, W not self-closing, hardware fails EP154; paper towels protrude- M 6” EP131, EP132, W 6.5” EP155, EP156

Women’s: lacks access symbol (checklist); toilet seat 16.5” aff EP150, EP151

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.12.1 through 1.12.10 is integral to compliance with title II program access test): (Restrooms - Multiple Users)

    - Remount signage for both on wall, latch side of the door, 48” to baseline of lowest character and 60” to baseline of highest character (EP57, EP120, EP133, EP157)

    - Lower mirrors in both so reflective surface is max 35” aff (EP122, EP123, EP143, EP144)

    - Insulate exposed pipes under one sink in both (EP124, EP145)

    - Lower paper towel dispensers in both to max 48” to highest operable part and relocate or recess to not interfere with general circulation path, protrusions can’t be greater than 4” (EP126, EP127, EP146, EP147, EP131, EP132, EP155, EP156)

    - Remount toilets in both to 16” to 18” from the side wall to centerline (EP128, EP148, EP149)

    - Remount side grab bars in both to max 12” from the rear wall at the close end and min 54" on the far end, and 33” to 36” aff (EP129, EP130, EP152, EP153)

    - Adjust stall doors in both to be self-closing (checklist)

    - Replace stall hardware in both with hardware operable without a tight pinch or grasp (EP125, EP154)

    - Acquire and mount signage, including access symbol for women’s (checklist)

    - Replace toilet seat in women’s, or re-set or replace toilet to 17” to 19” aff (EP150, EP151)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

[CHECKLIST-SINGLE USER] portable toilet door 74” EP401, 57” x 56” with 24” grab bars EP399, EP400, toilet paper and hand sanitizer at 55” EP396, EP397, EP398; lacks compliant signage (checklist)

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.12.1 is integral to compliance with title II program access test): (Restrooms - Single User)

    - Replace portable toilet with compliant model and place along AR (EP401, EP399, EP400, EP396, EP397, EP398, checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE TWO |

  - Other - not applicable

MAINTENANCE AND MARTINDALE ROAD BUILDINGS

  - Parking - see East Park

  - Exterior Accessible Route (includes common area and stairs)- no issues

  - Exterior Entry Signage - none present

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.3.1 and 1.3.2 is integral to compliance with title II program access test):

    - Mount signage at inaccessible entrances directing patrons in wheelchairs to accessible entrance (checklist)

    - Mount signage at entrance designating it as accessible (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE ONE |

  - Exterior Entry Doors - [CHECKLIST]

Maneuvering Clearance: maintenance: park office entry- table push side EP1; break room exit- 3.5% slope EP12, EP13; truck bay exit- small pad EP20; tractor garage w/ stairs- parking stop pull side, vehicle push side EP42, EP43; tractor garage- pad 35.5” deep on pull side EP51, EP52; martindale: door 2- storage push side (checklist), surface pull side EP29; door 1- storage push side EP33, surface pull side EP29

Change in level: maintenance: truck bay exit 5” EP22, EP23; tractor garage w/ stairs- .75” EP44, EP45; tractor garage- 1” EP53, EP54; martindale: door 2- 7.5” EP31, EP32; door 1- CIL EP34

Hardware: martindale: both doors have knobs EP30, (checklist)

LBF: maintenance: 4 of 5 doors exceed 8.5 lbf to open (checklist)

Closer: maintenance: 3 of 5 door close too fast (checklist)

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.4.1 through 1.4.9 is integral to compliance with title II program access test):

    - For doors along the public circulation route, relocate storage, furniture, and other obstacles to create 60” maneuvering space around doors (EP1, EP42, EP43, EP33, checklist)

    - For doors along the public circulation route, correct or repair slope at doorway landing to max 2.08% in any direction for level CFS (EP12, EP13)

    - For doors along the public circulation route, enlarge cement pad at exit to allow 18” clearance on the latch side, and extend a pathway away from the building for emergency egress (EP20, EP51, EP52)

    - For doors along the public circulation route, install cement pad at exit to allow 18” clearance on the latch side, and extend a pathway away from the building for emergency egress (EP29)

    - For doors along the public circulation route, repair, bevel, or ramp CIL at door entries to max .25” (EP22, EP23, EP44, EP45, EP53, EP54, EP31, EP32, EP34)

    - For doors along the public circulation route, replace hardware with lever hardware where indicated (EP30, checklist)

    - For doors along the public circulation route, inspect, adjust, and maintain 8.5 lbf to open exterior doors as a smart practice (checklist)

    - For doors along the public circulation route, inspect, adjust, and maintain closing speed on door closers so that doors do not close to 3” faster than 3 seconds when started at 70 degrees (checklist)

    - For employee only doors, make corrections upon renovation

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Elevator or Lift - not applicable

  - Interior Doors - [CHECKLIST]

Maneuvering Clearance: maintenance: park manager office- desk, chair pull side EP2; locker room- counter pull side EP3, lockers push side EP4; storage- wall 14” pull side EP5, EP6, no entry EP7; restroom- wall push side EP8; mechanical room- storage push side EP9; shop- wall 11” push side EP10, EP11; garage connector- vehicle on pull side EP46

Dimension (32"X80"): maintenance: restroom off shop- 26” EP14; parts- 27.5” EP16; garage connector- each leaf 27.5” EP47, EP48

Hardware: maintenance: 3 doors have knobs EP15, EP18, EP49; locker room to shop- pocket door hook EP19

LBF: maintenance: 4 of 4 doors exceed 5 lbf to open (checklist)

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.6.1 through 1.6.8 is integral to compliance with title II program access test):

    - For doors along the public circulation route, relocate storage, furniture, and other obstacles to create 60” maneuvering space around doors (EP2, EP3, EP4, EP9, EP46)

    - For doors along the public circulation route, widen entries to allow for 18” of maneuvering clearance on the pull side of doorways, in the alternative, remove door and frame and widen entry to 36” for access (EP5, EP6)

    - Leave as is, maneuvering clearance in closet or storage, correction is technically infeasible (EP7)

    - For doors along the public circulation route, remove and rehang door to open from the opposite side to create required 18” of maneuvering clearance on the pull side (EP8, EP10, EP11)

    - For doors along the public circulation route, replace doors with doors having 32" clear width (EP14, EP16, EP47, EP48)

    - For doors along the public circulation route, replace hardware with lever hardware where indicated (EP15, EP18, EP49, EP19)

    - For doors along the public circulation route, inspect, adjust, and maintain 5 lbf to open interior doors as a smart practice (checklist)

    - For employee only doors, make corrections upon renovation

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Interior Accessible Route (includes common area and stairs) - not applicable

  - Public Designated Use Spaces (includes classrooms, meeting rooms, special purpose rooms, and other spaces intended for use by the public) - [CHECKLIST]

Clear floor space: maintenance: break room- thermostat EP67

Reach Range: maintenance: break room- time clock EP65, thermostat 61.5” EP66, EP67

Sinks: maintenance: break room- 36.5” EP63, EP64

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.8.1 through 1.8.3 is integral to compliance with title II program access test):

    - Remove, or relocate storage in CFS at fixtures and operable parts (EP67)

    - Lower operating mechanisms to max 48” aff to the highest operable part (EP65, EP66, EP67)

    - Lower sink height to max 34” aff (EP63, EP64)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Employee Offices and Spaces - [CHECKLIST]

36” access aisle: maintenance: park manager- narrows at chair EP68; mechanical room- storage EP81; tractor garage- vehicle and equipment EP84; cold storage- obstructed by vehicles EP89

80” overhead: maintenance: tool/parts storage- heater 77.5” EP79, EP80; tractor garage with stairs- reduces under loft storage EP85, EP86

60” turning space: maintenance: park manager- furniture obstructs EP68; mechanical room- not large enough EP81

Protruding Objects: maintenance: shop- eyewash EP78; tractor garage- phone shelf EP82, EP83

Floor Surfaces: maintenance: tractor garage with stairs- stairs EP86; martindale: cold storage- loose stone floor EP96, CIL to stone floor EP96, stairs EP97

Clear Floor Space: martindale: cold storage- phone EP99

Reach Range: maintenance: locker room- hooks EP70, EP71; shop- alarm EP72, first aid kits EP73, EP74, EP76, EP77, phone EP75 ; tractor garage- phone EP82, blood kit EP82; martindale: cold storage- EP99

Operable Parts: maintenance: locker room- locker hardware EP69

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.9.1 and 1.9.2 is integral to compliance with title II program access test):

    - Employee only area permit approach, entry, and exit, relocate obstacles such as storage and equipment to create AR through rooms indicated (EP68, EP81, EP84, EP89)

    - Employee only areas permit approach, entry, and exit, relocate obstacles to create turning space of 60" in rooms indicated (EP68, EP81)

    - For all other deficits, leave as is, employee work area pursuant to 2010 Standards 106.5 Defined Terms, until an employee with a disability works here (EP69, EP70, EP71, EP72, EP73, EP74, EP75, EP76, EP77, EP78, EP79, EP80, EP82, EP83, EP85, EP86, EP86, EP96, EP97, EP99)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Assembly Areas - not applicable

  - Restrooms -  [CHECKLIST]

Main: lacks compliant signage EP100; lacks CFS at toilet, garbage can in forward space; centerline 13” EP103, EP104; side grab bar too small EP105, EP101; lacks rear grab bar EP101; flush on closed side EP102; toilet paper on back wall EP102; mirror 50” EP108; pipes lack insulation EP109; paper towel dispenser 58” EP106, EP107

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.11.1 through 1.11.9 is integral to compliance with title II program access test): (Main)

    - Acquire and mount signage, including access symbol, mounted on wall, latch side of door, 48” to baseline of lowest character and 60” to baseline of highest character (EP100)

    - Remove or relocate garbage can to allow forward CFS at toilet (EP101)

    - Replace side grab bar with a 42” long grab bar, mounted max 12” from the rear wall and 33” to 36” aff (EP105, EP101)

    - Acquire and mount 36" long rear grab bar mounted behind the toilet, 12” to one side of center and 24” to the other and 33” to 36” aff (EP101)

    - Replace toilet tank with one having flush mechanism on the open side, in the alternative, install an auto flush unit (EP102)

    - Remount toilet paper dispenser max 7” to 9” from front of toilet, 15” to 48” aff and min 12" above or 1.5" below grab bar on side wall (EP102)

    - Lower mirror so reflective surface is max 40” aff (EP108)

    - Insulate exposed pipes under sink (EP109)

    - Lower paper towel dispenser to max 48” to highest operable part (EP106, EP107)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

Shop: lacks compliant signage (checklist); side grab bar 36.5” aff EP113, EP114; toilet paper too far EP115; garbage can in sink CFS EP116; protruding objects- paper towels EP119, hand dryer EP117, EP118

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.11.1 through 1.11.5 is integral to compliance with title II program access test): (Shop)

    - Acquire and mount signage, including access symbol, mounted on wall, latch side of door, 48” to baseline of lowest character and 60” to baseline of highest character (checklist)

    - Remount grab bars in accessible stall to 33” to 36” aff and obstructions min 12" above or 1.5" below grab bar (EP113, EP114)

    - Remount toilet paper dispenser max 7” to 9” from front of toilet, 15” to 48” aff and min 12" above or 1.5" below grab bar on side wall (EP115)

    - Remove or relocate garbage can to allow forward CFS at toilet (EP116)

    - Relocate or recess towel dispenser and hand dryers to not interfere with general circulation path, protrusions can’t be greater than 4” (EP117, EP118)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Kitchen/Concessions - not applicable

  - Locker Rooms - not applicable

  - Aural and Visual Alarms - [CHECKLIST] none provided (checklist)

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.14.1 is integral to compliance with title II program access test):

    - Upon renovation install audible and visual alarms in all rooms and spaces (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Directional and Permanent Space Signs - none present

Recommendations (Englewood MetroPark is a site designated with accessible features so 1.15.1 through 1.15.3 is integral to compliance with title II program access test):

    - Create template for signs that addresses height of sign, size of characters, location of Braille, and other requirements (checklist)

    - Implement a sign revision program throughout the building, discriminating between directional signs and signs for permanent spaces (checklist)

    - Mount signage at all permanent rooms/spaces having Braille and the international symbol of accessibility, mounted at 60" to the middle of the sign and on the latch side of the door (checklist)

| Responsible Employee | Progress Towards Completion | Recommended Phase |
| --- | --- | --- |
| Richard V. Alder, Park Services Project Manager 937-278-2654 | (insert periodic notes regarding steps taken or work completed) | PHASE THREE |

  - Other - not applicable

FIVERIVERSMP ENGLWDMTROPRK SEC6