FINAL REPORT TO THE

Five Rivers MetroParks

Conclusions and

Recommendations

May 28, 2014

All Site Conclusions and Recommendations

Background

A step-by-step approach will help Five Rivers MetroParks understand and address the deficits in the individual site reports.  We recommend the following steps to accomplish the goal of making opportunities in MetroParks sites more accessible to people with disabilities.

A Guide to this Section

There are approximately 4,870 access deficits identified in the 42 site reports.  That is what is required by the ADA, the identification of every access deficit at every site and facility.  And, for every deficit, a solution must be identified.  Another way to consider this though is that for every deficit we found, we observed 10 elements that complied with the access requirements.  In other words, we saw approximately 40,000 access features that complied.

This section is all about the big picture.  As discussed in the cover letter with this report, the MetroParks does not necessarily have to make every site accessible.  It does have to make every program it conducts within its sites accessible.

We have attempted to identify some broad solutions, such as the refreshing of all accessible parking, as a way to address issues identified in the earlier 42 site reports, and as a way for MetroParks to better manage compliance.  This approach also gives MetroParks flexibility within its compliance efforts to move resources so that they are applied with optimal impact.  Consider these systemic changes as a complement to a site-by-site approach.

However, the scope of our work does not include the design of a solution.  Rather, it is performance based.  For example, if a park restroom needs to be made accessible we'll make that recommendation.  We will not design a solution that includes walls to be removed or plumbing solutions.  Those are tasks for MetroParks staff or contractors.

This is also about accountability.  The adjustments to door closers, eliminating changes in level, and other recommended actions are ineffective if not maintained over time by MetroParks employees.  We recommend the following to facilitate review:

First, read the final report cover letter to Richard V. Alder.  It describes the concepts and requirements invoked throughout the report.

Second, read this Conclusion section.  As mentioned above, this is a big picture review of the issues and solutions we recommend.

Third, read the 42 site reports.  Use your computer and you’ll have instant access to the report for that site, the photos, and the checklists.

Fourth, use your knowledge of the sites and of your staffs’ expertise.  You know Five Rivers MetroParks sites better than we do, and you certainly know the staff better than we do.  Blend in what you know with what we recommend in the report.

There is always another way to solve an access problem…perhaps you’ll be the one to see that solution.

Common Issues

In our evaluation, some common issues arose.  These included the way maintenance affects accessibility to playground surfaces used.  The common issues are also “big picture” items for MetroParks and incorporate many of the specific site recommendations.

Maintenance

MetroParks uses a conscientious staff to maintain its facilities and sites.  However, over time, every facility and site yields to wear and tear.  The recommendations below describe ways in which attention to maintenance can specifically address some access deficits.

- Provide training to maintenance staffs regarding the features of an accessible route and how to ensure that it remains unobstructed so that park amenities, e.g., garbage cans or signs, are placed adjacent to the accessible route.

- Provide training to staffs regarding the features of an accessible route and how to ensure that it remains unobstructed.

- Purchase some new tools.  MetroParks should have enough battery-powered digital levels, and tools to measure pounds of force for doors, to equip some staff for occasional spot-checks.  A great website for gauges is:

http://www.technologylk.com/crl-door-pressure-gauge-lk-HMC035.htm?src=froogle.

Changes in Level and Gaps

The routes and sidewalks that make up the MetroParks network of accessible routes are in fair condition.  Wear and tear, settling, weather, and other factors combine to cause changes in level and gaps along portions of those accessible routes, making that portion noncompliant and a barrier to many customers with physical and sensory disabilities.

Removing changes in level and gaps has a significant universal design benefit too, as more people with all types of conditions can more easily use MetroParks routes, such as staff pushing carts of supplies, parents with kids in strollers, and people using an assistive device such as a wheelchair, Segway, or walker.

- Eliminate changes in level by the end of 2015.  Using the rationale that the most severe changes in level are the greatest barriers to access, make changes in level of greater than .75” the highest priority.

Make changes in level of between .5” and .75” the second priority.  Make beveling of changes in level of .25” to .5” the third priority.

- Add change in level of more than .25” to park maintenance safety checklists in 2014.  This will help identify and correct these problems before they expand.  Make or buy pre-measured shims and distribute to employees for their use and ease of measurement.

- Add inspections for gaps of greater than .5” to park maintenance safety checklists in 2014.  Identify and fill these gaps before they expand.  In the alternative, consider resurfacing segments of deteriorated asphalt routes.

- Adopt a policy about the use of other Other Power Driven Mobility Devices in MetroParks facilities and at MetroParks sites, and promote that policy to the general public.  Every day, more people with limited physical mobility start to use a Segway or similar machines.

Pursuant to the new ADA title II regulation published September 14, 2010, this policy was to have been in place by March 15, 2011.

These assistive devices provide great benefits to people with disabilities and the sooner the MetroParks has a policy in regard to their use the better.  The policy could, at a minimum, address times of allowed use (dawn to dusk), speed limits, off-limits areas, status of the user as a person with a disability, and minimum age.

It is important to note that a power driven mobility device is not a wheelchair.  That device has a separate definition and is already allowed in facilities and parks.

The components of a policy are noted below.  The MetroParks is welcome to use some, all, or none of this, but a policy must be in place.  We recommend at least the following statements:

Definition: Other power-driven mobility device (OPDMD) means any mobility device powered by batteries, fuel, or other engines––whether or not designed primarily for use by individuals with mobility disabilities––that is used by individuals with mobility disabilities for the purpose of locomotion, including golf cars, electronic personal assistance mobility devices (EPAMDs), such as the Segway® PT, or any mobility device designed to operate in areas without defined pedestrian routes, but that is not a wheelchair within the meaning of this policy.

Definition: An electronic personal assistive mobility device (EPAMD) is a device used by a person with a mobility impairment for ambulation.  This definition does not include gasoline powered devices, golf cars, or riding lawn mowers.

Permission: The Five Rivers MetroParks authorizes persons with mobility impairments to use OPDMDs and EPAMDs in MetroParks facilities and sites subject to the following restrictions:

- The operator of the device must be a person with a mobility impairment, and upon request by MetroParks officials, shall produce proof of such within 72 hours;

- The device, if used in a facility or in a park, is allowed in any area of the facility or park in which the general public is allowed, with the exception of employee only spaces, stairways, and identified hazardous areas;

- The device, if used in a facility, must be controlled by the operator.  It:

- may not exceed 4 mph;

- shall be driven on the right side of the circulation route;

- is prohibited from carrying another person on the frame, or any object on the frame that may make the device less stable; and

- must not be operated in a dangerous or reckless manner that jeopardizes the safety of the operator, MetroParks employees, or MetroParks participants.

- The device, if used in a park or outside, must be controlled by the operator.  It:

- may not be operated between dusk and dawn unless equipped with headlights that are visible at 300’;

- may not exceed 6 mph;

- shall not be driven into wet or ecologically sensitive areas which are posted as such;

- shall be driven on the right side of the circulation route;

- is prohibited from carrying another person on the frame, or any object on the frame that may make the device less stable; and

- must not be operated in a dangerous or reckless manner that jeopardizes operator safety, MetroParks employees, or MetroParks participants.

- The MetroParks accepts no responsibility for storage of the device.

- The MetroParks accepts no liability for damage to the device, or injury to the operator, whether caused by the operator, another visitor to a MetroParks facility or site, or any other circumstance.

- MetroParks accepts no liability for damage caused by the operator of the device, or injury to others caused by the operator of the device.

- MetroParks reserves the right to suspend the use of facilities or sites by the operator if doing so is in the best interests of the MetroParks and its participants.

- MetroParks reserves the right to change, modify, or amend this policy at any time, as it would any other policy.

Obstructed Accessible Routes

Employees may see an accessible route as an empty 36” wide space in which a potted plant or garbage can is a perfect fit.  However, that blocks or obstructs the accessible route

- Provide training to park maintenance, recreation, and administration staffs regarding maintenance of accessible routes in parks and in recreation facilities.

Employee Work Areas

Five Rivers MetroParks employs many qualified and skilled full time staff, making parks and recreation services available to MetroParks residents.  MetroParks employs many more on a part-time or seasonal basis.  MetroParks likely already has employees with disabilities and in the future, will have more employees with disabilities, in all categories…full time, seasonal, and regular part time.

It is important to address access to work areas, and both the title II regulation and the work of the Access Board do so.  In section 203.9 of the 2010 Standards for Accessible Design, the treatment of employee areas is made clear.

Generally, a person with a disability should be able to approach, enter, and exit the work area.  This is addressed by requirements for accessible routes and accessible means of egress.  Other factors are door width, and threshold changes in level.

Excluded from this exception are several types of common spaces in employee areas.  Spaces such as the ones below must meet the access guidelines as they are excluded from the definition of employee-only areas:

- corridors;

- toilet rooms;

- kitchenettes for employee dining use, and

- break rooms.

In short, the key issues are the accessible route, changes in level, doors and entries, and maneuvering space once within the work area.  This approach is effective so long as when MetroParks hires an employee with a disability, or a current employee acquires a disability, it will remove architectural barriers in work areas or make other accommodations.

The two recommendations below are important for all employees at all MetroParks sites.

- Address accessibility in the MetroParks personnel policies, and note that, upon request by an employee, MetroParks will make reasonable accommodations, which may include the removal of architectural barriers in work spaces.

- Require new construction, and alterations or additions that include employee work areas to be designed and constructed so they are compliant with the 2010 Standards for Accessible Design.

Accessible Parking

MetroParks maintains approximately 2,215 standard parking spaces at sites, and 130 more that are designated as accessible parking stalls.

In correcting or refreshing its accessible stalls, MetroParks should address all of them at once to eliminate inconsistencies and come into compliance.

- Create a parking stall template.  A suggested template is below.

Parking Stall Dimensions

Stalls are a minimum of 8’ wide.  An adjacent access aisle must also be a minimum of 5’ wide.  The access aisle must be diagonally striped with high quality paint.

The collection of signs must include the US Department of Transportation R7-8 standard sign (the blue icon in a wheelchair).  Below that must be the statewide fine sign.  Unless the cities in the MetroParks jurisdiction have adopted a higher fine by ordinance, the sign must note the statewide fine.

Federal settlement agreements require a third sign, on at least one stall, that says VAN ACCESSIBLE.   This stall must be 11’ wide with a 5’ access aisle and can be shared with another accessible stall.  An acceptable alternate is 8’ and 8’.

Finally, the bottom edge of the R7-8 sign is a minimum of 60” above the finished grade.  We suggest that the signpost be located at the head of the accessible stall and that the curb cut and detectable warning run the distance of the access aisle.

The most common deficit in accessible parking stalls and access aisles is the slope.  The 2010 Standards limits the slope to not more than 2.08% in any direction.  This is a challenging requirement that can take considerable effort to meet.

Connection to the Accessible Route

The access aisles should connect to an accessible route.  The maximum running slope for the accessible route is 5%, and to account for heaving and settling, we recommend 4%.

The maximum cross slope is 2.08%.

Passenger Loading Zone

The loading zone must have an access aisle adjacent and parallel to vehicle pull-up space.  The loading zone access aisle must be a minimum of 60” wide and 20’ long.

Confirm this template to ensure compliant stalls.

- In 2015 implement a plan to correct or refresh every accessible stall at every MetroParks facility.  Incorporate this task into other plans that require parking lot repair, restriping, or resurfacing.

Running Slope and Cross Slope

We saw running slopes steeper than permitted.  At some sites this was a minimal issue, but at other sites it was a significant variance.  This condition naturally occurs when concrete settles, or when connections between new and old routes are off by fractions of an inch.  Cross slope is equally important, as it serves drainage as well as access purposes.

Site constraints at Five Rivers’ sites may pose challenges regarding the slope.  Do recall that it may be possible, on a limited basis, to apply the concept of technical infeasibility to existing routes at naturally occurring steeply sloped sites.  This is the exception not the rule.

- Adopt a policy that in new construction and alterations the slope of the AR shall not exceed 1:21, or 4.7%, as opposed to 1:20, or 5%.  This allows room for field error.

- Adopt a policy that in new construction and alterations the ramp slope shall not exceed 1:13, or 7.7%, as opposed to 1:12, or 8.33%.  This allows room for field error.

It also makes ramps easier to use for everyone, not just people with disabilities.  This universal design approach is also a risk management tool.

- Adopt a policy that in new construction or alterations the cross slope shall be an integral part of the project and shall not exceed 2% or 1:50.

Detectable Warnings

The US Access Board suspended the detectable warning requirement in the late 90’s, for a period of several years.  It was restored in 2002.  However, it is not required in the 2010 Standards.

We still, however, as a smart practice, recommend the use of detectable warnings.  It is typical to see noncompliant detectable warnings in every community.

The detectable warnings at curbs that are not compliant are often a cross-cut of concrete, or a grid laid on wet concrete to create a diamond-shaped indentation.  Over time these should be replaced.

- As with parking, develop a template for detectable warnings.

- In the same year that parking is refreshed, implement a plan to correct or refresh every detectable warning at every curb or crossing at MetroParks facilities.  If necessary, phase this out over a two or three year period.

Door Opening Force Requirements

In MetroParks buildings and facilities, there are approximately 596 doors.  Many have closer mechanisms.  Some of these need adjustment to bring the pounds of force (lbf) necessary into compliance (5 lbf for interior doors and 8.5 lbf (smart practice) for exterior doors).

However some of the closers are just old.  The wear and tear of 20 or more years erodes the closer effectiveness.

- Evaluate and determine the age of door closers.

- Add door closer maintenance checks to safety checklists in 2014 and for closers with 10 years of service or less, aggressively maintain them for effectiveness.

- Purchase and install new door closers for all exterior doors (with closers 20 years old or more) and 50% of interior doors in 2014 or as soon as is possible.

- Purchase and install new door closers for all remaining interior doors (with closers 20 years old or more) in 2015 or as soon as is possible.

Signage

MetroParks signs serve several purposes.  First, signs assist wayfinding in large buildings, such as Wegerzyn Gardens Visitor Center.  Second, signs identify important permanent elements of facilities, such as restrooms.  Third, signs facilitate access by people with vision and physical limitations.  We did not note a signage template at the sites we evaluated.

The Access Board treats two types of signs differently.  Signs for permanent spaces, such as a bathroom, must be in both Grade 2 Braille and raised lettering.  Signs that are directional or informational only require visual lettering of a certain size.  Be certain to incorporate these approaches into signs in buildings and sites operated by MetroParks.

- Develop a sign template in 2014 that describes where and in what facilities signs will be used.  The template could include size of sign, mounting height, mounting location, size of characters, space between characters, contrast between characters and background, icons or symbols used in the signs, MetroParks information in the signs (name of facility? phone number? main office number?), and more.

- Implement signage template and refresh MetroParks site signage in 2014.

Bathrooms

Bathrooms are an essential part of a visit to a Five Rivers MetroParks site.  Exercise, food and beverage, social activities, and more all rely on one of the oldest designs known to us.  Making those facilities accessible is tremendously important.

Additionally, portable toilets placed temporarily at sports fields and event venues must be accessible and must be served by an accessible route.

- Develop a bathroom template in 2014.  Confirm it with the State of Ohio.  Be sure to include temporary facilities such as portable toilets in the template.  The template should address the toilet itself; grab bars, items in the stall such as toilet paper and hooks, the stall, operating mechanisms, mirrors, sinks, hand towels, and more.

- Include bathroom renovations at facilities in the MetroParks Capital Acquisition and Replacement Plan.

- Consider the use of automatic flush controls.  These have environmental benefits and are also a great way to eliminate some accessibility problems.

- In the interim, implement non-structural modifications recommended in each section of this report, such as lowering mirrors, remounting grab bars, changing the height of toilets and urinals, installing compliant stall hardware, and so forth.  These less costly changes on a site-by-site basis will serve your customers well until resources are available to renovate restrooms on a comprehensive scale.

- Make one portable toilet, if one is provided at a site, accessible.  This includes a portable toilet placed at a picnic shelter or adjacent to sports fields.  These must be accessible and must be served by an accessible route.

The MetroParks has sites with portable toilets; this must be addressed. Use our single-user toilet checklist, and require compliance by MetroParks vendors.

Lockers and Locker Rooms

Research shows that people with disabilities will refrain from using public facilities if they fear that sites are not accessible or they’ll be embarrassed in their attempt to use sites.

We all know the benefits of recreation participation.  If a person with a physical disability comes to Riverscape MetroPark Bike Hub to bike, there are not enough designated accessible lockers and no fully compliant showers.

- Implement the locker room changes for the Riverscape Bike Hub

- Promote availability of accessible locker rooms once completed.  Announce these changes to the community and see more participation by people with disabilities.

Alarms

In existing facilities where an aural or audible fire alarm system is provided, a visual alarm is not required unless the building was constructed after January 26, 1992 or has been upgraded since that same date.

If an alarm in an existing facility is audible only, it need not be modified to include a visual alarm unless it is replaced or upgraded in the future.

- Determine in 2014 if systems have been upgraded or replaced since 1992.

- Develop a plan in 2014 for the installation of aural and visual alarms in renovations.

- Retrofit construction that has occurred since 1992 to include aural and visual alarms by the end of 2016.

Brochures

The use of a park grid in the MetroParks brochures is an important tool for residents and can now be used to communicate about accessibility.  Create one to incorporate the access work MetroParks staff completes and indicate in your grid where, for example, the accessible picnic areas are, or where the accessible playgrounds are.

- Update the parks and facilities information on the website to reflect MetroParks plans regarding our recommendations, and to note which sites are accessible or will be made accessible.

Website

The title II regulation requires that all public communication used by the MetroParks be available to people with disabilities.  Many people with vision impairments use websites every day with the aid of technical equipment.

MetroParks is required to evaluate its website and make necessary changes so that the website can be read by that type of equipment.

A link at the US Department of Justice website offers guidance on this.  The MetroParks IT staff should become familiar with this issue.  Go to http://www.ada.gov/websites2.htm.

MetroParks can also check the accessibility of its website at a free service.  Link to Cynthia Says at www.contentquality.com/ and test your website.

- Evaluate the MetroParks website and make changes so that the information on the site is accessible to people with disabilities.

Maintenance Buildings

In individual site reports we address the Maintenance Buildings.  As noted earlier, Five Rivers can apply a different standard to spaces used only as employee work areas.  MetroParks maintenance staff should receive training in regard to the application of the approach, enter, and exit strategy so that they understand the reason for the various requirements.

- Train maintenance staff supervisors in accessibility concepts that are applicable to the maintenance building.

- Implement recommendations regarding parking, accessible route, changes in level, gaps, doors, and alarm systems at the Maintenance Buildings.

Playgrounds

The minimum required of the MetroParks by title II of the ADA is that the “program” of playgrounds be accessible to residents.  This is measured by the “program access test” described in section 35.150 of the title II regulation (see 28 CFR Part 35).

For similar multiple sites, no guidance is given as to how many existing playgrounds should be made accessible.  Again, a good practice is to treat this as a planning exercise and aim for 1 of 3 playgrounds being made accessible.

Our evaluation included 6 playgrounds.  Of these, two are accessible.  We believe one more could be made accessible with modest changes.  Any new playgrounds to be replaced must comply with the 2010 Standards and will therefore be accessible.

The Program Access Chart, along with Five Rivers MetroParks Playground Map at the end of this section, illustrates the areas where work is recommended so that every resident of the MetroParks is close to an accessible playground. [MetroParks Playground Map]

- Make corrections cited in the reports so the playgrounds below remain accessible:

- Hills and Dales MetroPark

- Wesleyan MetroPark

- Make the corrections so the playgrounds at sites below become accessible:

- Island MetroPark (small 5 to 12)

- Leave as is the playgrounds at the parks named below, and if future alterations or renovations occur at those sites, make them accessible.

- Eastwood MetroPark

- Island MetroPark (large 5 to 12, 2 to 5)

- Advertise the accessible playgrounds in the MetroParks website and publications.

Fishing

The minimum required of the MetroParks by title II of the ADA is that the “program” of fishing be accessible to residents.  This is measured by the “program access test” found in section 35.150 of the title II regulation (see 28 CFR Part 35).  For similar multiple sites, no guidance is given as to how many existing fishing areas should be accessible.

We recommend that a minimum of one area of every three be accessible.  We saw 13 fishing areas and three are accessible.  We recommend access to one more.

The Program Access Chart at the end of this section, along with the Five Rivers MetroParks Fishing Map, illustrates the areas where work is recommended so that every resident is close to an accessible fishing area. [MetroParks Fishing Map]

- Make corrections cited in the reports so the fishing areas below remain accessible:

- Carriage Hill MetroPark (1 of 2)

- Possum Creek MetroPark (1 of 7)

- Twin Creek MetroPark

- Make corrections cited in the report so the fishing areas below becomes accessible:

- Taylorsville MetroPark

- Leave as is the fishing areas at the following sites:

- Carriage Hill MetroPark (1 of 2)

- Englewood MetroPark

- Possum Creek MetroPark (6 of 7)

- Sunrise MetroPark

- Advertise the accessible fishing areas in MetroParks website and publications.

Bow Hunting

The minimum required of the MetroParks by title II of the ADA is that the “program” of bow hunting be accessible to residents.  This is measured by the “program access test” found in section 35.150 of the title II regulation (see 28 CFR Part 35).

For similar multiple sites, no guidance is given as to how many existing bow hunting areas should be accessible.  We recommend that a minimum of one of every three be accessible.

We saw 8 areas and none are accessible.  We recommend access to at least two.

The Program Access Chart at the end of this section, along with the Five Rivers MetroParks Bow Hunting Map, illustrates accessible sites so that every resident is close to an accessible bow hunting area. [MetroParks Bow Hunting Map]

- Make corrections cited in the reports so the bow hunting areas below become accessible:

- Medlar Conservation Area (1 of 2)

- Upper Twin Conservation Area (1 of 3)

- Leave as is the bow hunting areas at the following sites:

- Medlar Conservation Area (1 of 2)

- Shiloh Woods Conservation Area (2 of 2)

- Shoup Mill Conservation Area

- Upper Twin Conservation Area (2 of 3)

- Advertise the accessible bow hunting areas in the MetroParks website and publications.

Boat Launch

The minimum required of the MetroParks by title II of the ADA is that the “program” of boat launches be accessible to residents.  This is measured by the “program access test” found in section 35.150 of the title II regulation (see 28 CFR Part 35).

For similar multiple sites, no guidance is given as to how many existing boat launches should be accessible.  We recommend that at least one of every three be accessible.

There are 12 boat launches; three are accessible.  We recommend access to one more.

The Program Access Chart at the end of this section, along with the Five Rivers MetroParks Boat Launch Map, illustrates the areas where work is recommended so that every resident of the MetroParks is close to an accessible boat launch. [MetroParks Boat Launch Map]

- Make corrections cited in reports so the boat launches below remain or become accessible:

- Eastwood MetroPark (1 of 5)

- Englewood MetroPark (1 of 2)

- Germantown MetroPark

- Island MetroPark

- Leave as is the boat launches at the following sites:

- Aullwood MetroPark

- Eastwood MetroPark (4  of 5)

- Englewood MetroPark (1 of 2)

- Huffman MetroPark

- Taylorsville MetroPark

- Advertise the accessible boat launches in MetroParks website and publications.

Camping

The minimum required of the MetroParks by title II of the ADA is that the “program” of camping be accessible to residents.  This is measured by the “program access test” found in section 35.150 of the title II regulation (see 28 CFR Part 35).

For similar multiple sites, no guidance is given as to how many existing campgrounds should be accessible.  We recommend that at least one of every three be accessible.

The issue of camping access is not yet settled as a final and enforceable standard.  There is significant guidance from the US Access Board, and we have applied it here to MetroParks camping areas.  However the US Access Board does not have the authority to establish a Standard, which is the step above the final guideline that exists today.

That said, we recommend MetroParks continue as a smart practice to make and maintain camping areas in compliance with the Access Board guidance.  There are 18 campsites and none are accessible.  We recommend access to 5 sites.

The Program Access Chart at the end of this section, along with the Five Rivers MetroParks Campground Map, illustrates the areas where work is recommended so that every resident of the MetroParks is close to an accessible campground. [MetroParks Campground Map]

- Make corrections cited in reports so the campgrounds below become accessible:

- Englewood MetroPark (1 of 4)

- Germantown MetroPark (2 of 5)

- Sugar Creek MetroPark (1 of 2)

- Taylorsville MetroPark

- Leave as is the campgrounds at the following sites:

- Carriage Hill MetroPark

- Englewood MetroPark (3 of 4)

- Germantown MetroPark (3 of 5)

- Possum Creek MetroPark (2)

- Sugar Creek MetroPark (1 of 2)

- Twin Creek MetroPark (3)

- Advertise the accessible campgrounds in MetroParks website and publications.

Overlooks

The minimum required of the MetroParks by title II of the ADA is that the “program” of overlooks be accessible to residents.  This is measured by the “program access test” found in section 35.150 of the title II regulation (see 28 CFR Part 35).

For similar multiple sites, no guidance is given as to how many existing overlooks should be accessible.  We recommend that at least one of every three be accessible.

There are 8 overlooks and 5 are accessible.  We recommend no new access.

The issue of overlook access is not yet settled as a final and enforceable standard.  There is significant guidance from the US Access Board, and we have applied it here to MetroParks overlooks.  However the US Access Board does not have the authority to establish a Standard, which is the step above the guideline that exists today.

That said, we recommend the MetroParks continue as a smart practice to make and maintain overlooks so that they are accessible.

The Program Access Chart at the end of this section, along with the Five Rivers MetroParks Overlooks Map, illustrates accessible trails so that every resident of the MetroParks is close to an accessible overlook. [Metroparks Overlooks Map]

- Make corrections cited in reports so the overlooks remain accessible:

- Carriage Hill MetroPark

- Cox Arboretum

- Deeds Point MetroPark

- Germantown MetroPark

- Hills and Dales MetroPark

- Leave as is the overlooks at the following sites:

- Taylorsville MetroPark (3)

- Advertise the accessible overlooks in MetroParks website and publications.

Trails/Bikeways

The minimum required of the MetroParks by title II of the ADA is that the “program” of trails and bikeways be accessible to residents.  This is measured by the “program access test” found in section 35.150 of the title II regulation (see 28 CFR Part 35).

For similar multiple sites, no guidance is given as to how many existing trails and bikeways must be accessible.  We recommend that at least one of every three be accessible.  There are 74 trails and bikeways and 10 are accessible.  We recommend access to 8 more.

The issue of trail access is not yet settled as a final and enforceable standard.  There is significant guidance from the US Access Board, and we have applied it here to MetroParks trails and bikeways.

However the US Access Board does not have the authority to establish a Standard, which is the step above the final guideline that exists today.

That said, we recommend MetroParks continue as a smart practice to make and maintain trails so that they can achieve 30% of the total trail mileage to be accessible.

We also note here that six alternates addressed portions of trails.  These are highlighted in the trails or bikeways site reports.  The alternates are:

Alternate 1 addressed designated portions of The Great Miami Bikeway (Route 25) and The Mad River Bikeway (Route 8).

Alternate 2 addressed designated portions of The Wolf Creek Bikeway (Route 38).

Alternate 3 addressed designated portions of The Stillwater River Bikeway (Route 7).

Alternate 4 addressed designated portions of The Iron Horse Bikeway (Route 9).

Alternate 5 addressed the 4.5 mile portion of The Wolf Creek Bikeway (Route 38).

Alternate 6 addressed the 2 mile portion of The Wolf Creek Bikeway (Route 38).

The Program Access Chart at the end of this section, along with the MetroParks Trails Map, illustrates accessible trails so that every resident of the MetroParks is close to an accessible trail or bikeway. [MetroParks Trails Map]

- Make corrections cited in reports so the trails below remain accessible:

- Englewood MetroPark (1 of 8)

- Germantown MetroPark (1 of 8)

- Hills and Dales MetroPark (1 of 3)

- Sugar Creek MetroPark (1 of 5)

- Great Miami Bikeway

- Mad River Bikeway

- Stillwater River Bikeway

- Creekside Bikeway

- Iron Horse Bikeway

- Wolf Creek Bikeway

- Make corrections cited in reports so the trails below become accessible:

- Aullwood MetroPark

- Carriage Hill MetroPark (2 of 4)

- Dull Woods Conservation Area

- Possum Creek MetroPark (2 of 7)

- Wegerzyn Gardens (1 of 2)

- Woodman Fen Conservation Area

- Leave as is the trails at the following sites:

- Carriage Hill MetroPark (2 of 4)

- Cox Arboretum (3)

- Englewood MetroPark (7 of 8)

- Germantown MetroPark (7 of 8)

- Hills and Dales MetroPark (2 of 3)

- Huffman MetroPark

- Possum Creek MetroPark (5 of 7)

- Sugar Creek MetroPark (4 of 5)

- Taylorsville MetroPark (6)

- Twin Creek MetroPark (9)

- Wegerzyn Gardens (1 of 2)

- Mitigation Bank Conservation Area

- River Ridge Conservation Area (2)

- MoMBA Bike Area (6)

- Advertise the accessible trails and bikeways in MetroParks website and publications.

Picnic Areas

The minimum required of the MetroParks by title II of the ADA is that the “program” of picnicking be accessible to residents.  This is measured by the “program access test” described in section 35.150 of the title II regulation (see 28 CFR Part 35).

For similar multiple sites, no guidance is given as to how many existing picnic sites should be accessible.  There 61 picnic areas and 25 are accessible.  We recommend access to 7 more.  Many of these sites need tables or other minor corrections.

The issue of picnic area access is not yet settled as a final and enforceable standard.  There is significant guidance from the US Access Board, and we have applied it here to MetroParks picnic areas.  However the US Access Board does not have the authority to establish a Standard, which is the step above the final guideline that exists today.

That said, we recommend MetroParks continue as a smart practice to adhere to the Access Board guidance on this matter.

The Program Access Chart at the end of this section, with the MetroParks Picnic Areas Map, illustrates accessible picnic areas so that every resident of the MetroParks is close to an accessible picnic area. [MetroParks Picnic Areas Map]

- Make corrections needed to maintain access, including adding tables, to picnic areas at:

- Carriage Hill MetroPark (2 of 6)

- Eastwood MetroPark (2 of 5)

- Englewood MetroPark (2 of 5)

- Germantown MetroPark (4 of 6)

- Hills and Dales MetroPark (3 of 5)

- Island MetroPark (5)

- Possum Creek MetroPark (5 of 8)

- Weslyan MetoPark (2)

- Make corrections to create access, including adding tables, to picnic areas at:

- Englewood MetroPark (1 of 5)

- Taylorsville MetroPark (6 of 8)

- Leave as is the picnic areas at the sites below:

- Carriage Hill MetroPark (4 of 6)

- Eastwood MetroPark (3 of 5)

- Englewood MetroPark (2 of 5)

- Germantown MetroPark (2 of 6)

- Hills and Dales MetroPark (2 of 5)

- Huffman MetroPark (7)

- Possum Creek MetroPark (3 of 8)

- Sugar Creek MetroPark

- Taylorsville MetroPark (2 of 8)

- Twin Creek MetroPark

- Wegerzyn Gardens (2)

- Advertise accessible picnic areas in the MetroParks website and publications.

Public Feedback

An integral part of the self-evaluation of sites and facilities, and the development of a transition plan, is the involvement of the public.

A public forum was conducted on Thursday, May 1 at Possum Creek MetroPark.  Although few attended, the feedback in the meeting was excellent and addressed several emerging questions by MetroParks staff.

Transition Plan

The MetroParks must have a transition plan per 35.150(d) of the DOJ title II regulation.  The plan should identify the barrier, the corrective work, the date by which the work will occur (in our reports, the Phase), and the person responsible for barrier removal.

Barriers should be removed as soon as is possible.  Phasing the work to be done allows for access to occur and makes the best use of the resources of the Fiver Rivers MetroParks.

In the view of DOJ, the recreation design requirements were available to the MetroParks since 2004, if not earlier.  Enforcement staff has said at meetings and in conversations that work should have already been underway to identify and remedy access deficits.

We recommend work in three phases.  We also note the work we recommend need not occur in a category titled MetroParks Option.  Should the MetroParks plans change, or should other resources become available, the corrective work needed at these sites is already known.  We acknowledge that some phases, perhaps Phase One, require two or perhaps three fiscal years for completion.

We have made cost projections for the corrective work recommended.  We note that these are not estimates and should be used only as a guide.  The year in which the work will occur, the relationship with the contractor, and many other factors must be considered.

The total of corrective work we recommend is $2,942,008.75

In Phase One, we recommend work in the amount of $1,212,722.00.  Generally the work in this Phase falls into two categories: easy to do with existing staff and resources (low-hanging fruit), and old requirements (such as parking) at sites otherwise accessible.

Because of the size of this group, and to create phases that are more equal to each other, we have moved some facility work to Phase Two and Phase Three.

In Phase Two, we recommend work in the amount of $1,152,421.00.  Generally the work in this Phase includes changes to recreation amenities, such as playground surfaces, boating areas, and fishing areas.

In Phase Three, we recommend work in the amount of $576,865.75.  Generally the work in this Phase falls into two categories: elements not yet addressed by a final Standard, such as trails, and elements where correction is complex or costly.

We identified work in the amount of $146,068.50 in MetroParks Option.  Generally this is an element or site with access deficits where we believe MetroParks already meets the program access test and need not make these sites accessible, until later altered for another purpose.

Conclusion

Five Rivers MetroParks has a variety of recreation facilities and sites.  The skilled staff operates facilities and sites the community wants and enjoys.  This report identifies some issues that are typical of an outdoor recreation infrastructure. MetroParks takes steps towards accessibility every year and that undoubtedly helped.

MetroParks should determine to what extent it will act on our recommendations and any staff recommendations.  Access work should occur every year during the transition plan.

While no one can say with certainty how long MetroParks should feel comfortable in stretching these projects, we usually suggest it be not more than a 3 year range after the effective date of the 2010 Standards…that means March 15, 2015.  As the MetroParks has just finished this access audit, we suggest December 31, 2017 for a quickly-paced deadline.

Department of Justice officials have said work must be completed as soon as is possible.  Acting in good faith by MetroParks is critical.  Be certain to understand that MetroParks could be forced to accelerate its pace.  Making access work a high priority is critical.

Your strategy should definitely address the common issues identified in this report.

Five Rivers MetroParks should be commended for undertaking this task.  Although this access audit and the transition plan are both mandated tasks, many of your neighbors have not completed these steps.

In closing, thanks again to the staff at Fiver Rivers MetroParks for their cooperation and spirit.  All of the team at our firm enjoyed working with them.  We acknowledged Richard V. Alder for his assistance earlier, and we do so again here.

Call me at Recreation Accessibility Consultants at 224/293-6451 if there are any questions.  Thanks again for inviting us to work with the MetroParks.

Submitted by	__________________________________________

John N. McGovern, J.D., President

Recreation Accessibility Consultants LLC

FIVERIVERS METROPARKS CONCLUSIONS 201401